|  | Simulation of 9054T115_Steel Workbench Top_w_installation_holes Date: 03 October 2025 Designer: Solidworks Study name: Static 3 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 3 Study Properties 4 Units 4 Material Properties 5 Loads and Fixtures 6 Connector Definitions 7 Interaction Information 7 Mesh information 8 Sensor Details 9 Resultant Forces 10 Beams 10 Study Results 11 Conclusion 14. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: 9054T115_Steel Workbench Top_w_installation_holes Current Configuration: 9054T115. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. LPattern1; Solid Body; Mass:0.996488 kg Volume:0.000124561 m^3 Density:8,000 kg/m^3 Weight:9.76558 N; C:\Users\Lucas\Documents\Projects\Co_Printing\RAAM\Layout\9054T115_Steel Workbench Top_w_installation_holes.SLDPRT Oct 3 10:49:03 2025. Fillet3; Solid Body; Mass:0.996488 kg Volume:0.000124561 m^3 Density:8,000 kg/m^3 Weight:9.76558 N; C:\Users\Lucas\Documents\Projects\Co_Printing\RAAM\Layout\9054T115_Steel Workbench Top_w_installation_holes.SLDPRT Oct 3 10:49:03 2025. Cut-Extrude8; Solid Body; Mass:35.9309 kg Volume:0.00449136 m^3 Density:8,000 kg/m^3 Weight:352.122 N; C:\Users\Lucas\Documents\Projects\Co_Printing\RAAM\Layout\9054T115_Steel Workbench Top_w_installation_holes.SLDPRT Oct 3 10:49:03 2025.

# Study Properties Study name; Static 3. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\Lucas\Documents\Projects\Co_Printing\RAAM\Layout).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; AISI 316 Stainless Steel Sheet (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 1.72369e+08 N/m^2. Tensile strength:; 5.8e+08 N/m^2. Elastic modulus:; 1.93e+11 N/m^2. Poisson's ratio:; 0.27. Mass density:; 8,000 kg/m^3. Thermal expansion coefficient:; 1.6e-05 /Kelvin.; SolidBody 2(LPattern1)(9054T115_Steel Workbench Top_w_installation_holes), SolidBody 3(Fillet3)(9054T115_Steel Workbench Top_w_installation_holes), SolidBody 5(Cut-Extrude8)(9054T115_Steel Workbench Top_w_installation_holes). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 4 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -0.0703937; 1,457.99; -0.000139266; 1,457.99. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Remote Load (Distributed connection)-2; Entities:; 1 face(s). Connection Type:; Distributed. Weighting Factor:; Default (Constant). Coordinate System:; Global cartesian coordinates. Translational Components:; ---,-543 N,---. Rotational Components:; ---,---,---. Reference coordinates:; -715 473.15 0 mm.. Remote Load (Distributed connection)-1; Entities:; 1 face(s). Connection Type:; Distributed. Weighting Factor:; Default (Constant). Coordinate System:; Global cartesian coordinates. Translational Components:; ---,-543 N,---. Rotational Components:; ---,---,---. Reference coordinates:; 715 473.15 0 mm.. Gravity-1; Reference:; Top Plane. Values:; 0 0 -9.81. Units:; m/s^2..

# Connector Definitions No Data

# Interaction Information Interaction; Interaction Image; Interaction Properties. Component Interaction-2; Type:; Bonded. Components:; 3 Solid Body (s). Options:; Independent mesh..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 1.50744 in. Minimum element size; 0.075372 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 2203136. Total Elements; 1286920. Maximum Aspect Ratio; 43.764. % of elements with Aspect Ratio < 3; 90.7. Percentage of elements with Aspect Ratio > 10; 0.254. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:01:15. Computer name:; IP-5-FLEX. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-1; Entities:; 3 Solid Body (s). Units:; mm. Size:; 15. Ratio:; 15.. Control-2; Entities:; 16 edge(s). Units:; mm. Size:; 4. Ratio:; 4.. ## Mesh Quality Plots Name; Type; Min; Max. Quality1; Mesh; -; -. 9054T115_Steel Workbench Top_w_installation_holes-Static 3-Quality-Quality1.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.0703937; 1,457.99; -0.000139266; 1,457.99. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -4.7892; 263.308; -2.98509; 263.368. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 2.779e+01N/m^2 Node: 172216; 1.197e+08N/m^2 Node: 238999. 9054T115_Steel Workbench Top_w_installation_holes-Static 3-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 12851; 3.463e+00mm Node: 1988448. 9054T115_Steel Workbench Top_w_installation_holes-Static 3-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 1.445e-10 Element: 1067586; 4.158e-04 Element: 1249859. 9054T115_Steel Workbench Top_w_installation_holes-Static 3-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Automatic; 1.441e+00 Node: 238999; 3.000e+00 Node: 1. 9054T115_Steel Workbench Top_w_installation_holes-Static 3-Factor of Safety-Factor of Safety1.

# Conclusion